Visual C++ code

void CTripodDlg::doMyImageProcessing(LPBITMAPINFOHEADER lpThisBitmapInfoHeader)

{

// doMyImageProcessing:  This is where you'd write your own image processing code

// Task: Read a pixel's grayscale value and process accordingly

unsigned int	W, H;			// Width and Height of current frame [pixels]

unsigned int    row, col;		// Pixel's row and col positions

unsigned long   i;			// Dummy variable for row-column vector

unsigned int	max_row;		// Row of the brightest pixel

unsigned int	max_col;		// Column of the brightest pixel

BYTE		max_val = 0;	        // Value of the brightest pixel

// Values used for calculating range from captured image data

// these values are only for a specific camera and laser setup

const double	gain = 0.0024259348;	// Gain Constant used for converting

// pixel offset to angle in radians

const double	offset = -0.056514344;	// Offset Constant

const double	h_cm = 5.842;		// Distance between center of camera and laser

double		range;		        // Calculated range

unsigned int	pixels_from_center;	// Brightest pixel location from center

// not bottom of frame

char		str[80];	        // To print message

CDC		*pDC;			// Device context need to print message

W = lpThisBitmapInfoHeader->biWidth;	// biWidth: number of columns

H = lpThisBitmapInfoHeader->biHeight;	// biHeight: number of rows

for (row = 0; row < H; row++) {

for (col = 0; col < W; col++) {

// Recall each pixel is composed of 3 bytes

i = (unsigned long)(row*3*W + 3*col);

// If the current pixel value is greater than any other,

// it is the new max pixel

if (*(m_destinationBmp + i) >= max_val)

{

max_val = *(m_destinationBmp + i);

max_row = row;

max_col = col;

}

}

}

// After each frame, reset max pixel value to zero

max_val = 0;

for (row = 0; row < H; row++) {

for (col = 0; col < W; col++) {

i = (unsigned long)(row*3*W + 3*col);

// Draw a white cross-hair over brightest pixel in the output display

if ((row == max_row) || (col == max_col))

*(m_destinationBmp + i) =

*(m_destinationBmp + i + 1) =

*(m_destinationBmp + i + 2) = 255;

}

}

// Calculate distance of brightest pixel from center rather than bottom of frame

pixels_from_center = 120 - max_row;

// Calculate range in cm based on bright pixel location, and setup specific constants

range = h_cm / tan(pixels_from_center * gain + offset);

// To print message at (row, column) = (75, 580)

pDC = GetDC();

// Display frame coordinates as well as calculated range

sprintf(str, "Max Value at x= %u, y= %u, range= %f cm    ",max_col, max_row, range);

pDC->TextOut(75, 580, str);

ReleaseDC(pDC);

}