Visual basic code

Private Sub exit_Click()

' only if running...

If (Timer1.Enabled) Then

Timer1.Enabled = False  'Stop Timer

VideoOCX.Stop

VideoOCX.Close

End If

End

End Sub

Private Sub Start_Click() 'Init VideoOCX Control, allocate memory and start grabbing

If (Not Timer1.Enabled) Then

Start.Caption = "Stop"

' Disable internal error messages in VideoOCX

VideoOCX.SetErrorMessages False

' Init control

If (Not VideoOCX.Init) Then

' Init failed. Display error message and end sub

MsgBox VideoOCX.GetLastErrorString, vbOKOnly, "VideoOCX Error"

End

Else

' Allocate memory for global image handle

capture_image = VideoOCX.GetColorImageHandle

' result_image = VideoOCX_Processed.GetColorImageHandle

Timer1.Enabled = True 'Start capture timer

' Start Capture mode

If (Not VideoOCX.Start) Then

' Start failed. Display error message and end sub

MsgBox VideoOCX.GetLastErrorString, vbOKOnly, "VideoOCX Error"

End

End If

End If

Else

Start.Caption = "Start"

Timer1.Enabled = False  'Stop Timer

VideoOCX.Stop

VideoOCX.Close

End If

End Sub

Private Sub Timer1_Timer()

' Timer for capturing - handles videoOCXTools

Dim matrix As Variant

Dim height, width As Integer

Dim r, c As Integer

Dim max_r, max_c As Integer

Dim max_red As Integer

Dim gain, offset As Variant

Dim h_cm As Variant

Dim range As Integer

Dim pixels_from_center As Integer

' Calibrated parameter for pixel to distance conversion

gain = 0.0024259348

offset = -0.056514344

h_cm = 5.842

max_red = 0

' Capture an image

If (VideoOCX.Capture(capture_image)) Then

' VideoOCX.Show capture_image

' Matrix transformation initialization

matrix = VideoOCX.GetMatrix(capture_image)

height = VideoOCX.GetHeight

width = VideoOCX.GetWidth

' Image processing code

' The laser dot should not be seen above the middle row (with a little pad)

For r = height / 2 - 20 To height - 1

' Our physical setup is roughly calibrated to make the laser

' dot in the middle columns...dont bother lookng too far away

For c = width / 2 - 25 To width / 2 + 24

' Look for the largest red pixel value in the scene (red laser)

If (matrix(c, r, 2) > max_red) Then

max_red = matrix(c, r, 2)

max_r = r

max_c = c

End If

Next c

Next r

' Calculate the distance for the laser dot from middle of frame

pixels_from_center = max_r - 120

' Calculate range in cm based on calibrated parameters

range = h_cm / Tan(pixels_from_center * gain + offset)

' Print laser dot position row and column to screen

row_val.Caption = max_r

col_val.Caption = max_c

' Print range to laser illuminated object to screen

range_val.Caption = range

' Draw a red vertical line to intersect target

For r = 0 To height - 1

matrix(max_c, r, 2) = 255

Next r

' Draw a red horizontal line to intersect target

For c = 0 To width - 1

matrix(c, max_r, 2) = 255

Next c

VideoOCX.ReleaseMatrixToImageHandle (capture_image)

End If

VideoOCX.Show capture_image

End Sub